C      RAY FAILURE CODE DOCUMENTATION

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'RAY DID NOT INTERSECT THE SURFACE'

C        RAYCOD(1)=1

C        RAYCOD(2)=I

C

C

C

C        'RAY FAILURE OCCURRED AT SURFACE ',J

C        'CONVERGENCE FAILURE OCCURRED DURING RAY INTERSECTION'

C        'WITH ASPHERIC, TORIC OR SPECIAL SURFACE'

C        'ACCURACY OF INTERSECTION WAS = ',LENG

C        RAYCOD(1)=2

C        RAYCOD(2)=J

C

C

C

C        'RAY FAILURE OCCURRED AT SURFACE ',NEWREF

C        'RAY FAILED TO CONVERGE TO REFERENCE SURFACE RAY-AIM POINT'

C        RAYCOD(1)=3

C        RAYCOD(2)=NEWREF

C

C

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'TOTAL INTERNAL REFLECTION'

C        RAYCOD(1)=4

C        RAYCOD(2)=I

C

C

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'ANGLE OF DIFFRACTION AT THE GRATING AT SURFACE

C        IS PHYSICALLY UNREALIZABLE'

C        RAYCOD(1)=5

C        RAYCOD(2)=I

C

C

C

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'RAY BLOCKED BY CIRCULAR CLEAR APERTURE'

C        RAYCOD(1)=6

C        RAYCOD(2)=I

C

C

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'RAY BLOCKED BY CIRCULAR OBSCURATION'

C        RAYCOD(1)=7

C        RAYCOD(2)=I

C

C

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'CHIEF RAY FROM CURRENT OBJECT POINT COULD NOT BE TRACED'

C        RAYCOD(1)=8

C        RAYCOD(2)=NEWOBJ

C

C

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'ANGLE OF DIFFRACTION AT THE HOE SURFACE IS PHYSICALLY

C        UNREALIZABLE'

C        RAYCOD(1)=9

C        RAYCOD(2)=I

C

C         RAY FAILURE CODE 10 RESERVED FOR FUTURE USE

C

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'REFERENCE RAY CAN NOT BE TRACED AT ZERO WAVELENGTH ',I

C        RAYCOD(1)=11

C        RAYCOD(2)=I

C

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'RAY CAN NOT BE TRACED AT ZERO WAVELENGTH ',I

C        RAYCOD(1)=12

C        RAYCOD(2)=I

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'RAY MISSED GRAZING INCIDENCE SURFACE SECTION'

C        RAYCOD(1)=13

C        RAYCOD(2)=I

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'ILLUMINATION RAY BLOCKED BY CLEAR APERTURE'

C        RAYCOD(1)=14

C        RAYCOD(2)=I

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'NO GRID FILE EXISTS FOR THIS SURFACE'

C        'OR IT HAS INSUFFICIENT DATA FOR THIS SURFACE'

C        RAYCOD(1)=15

C        RAYCOD(2)=I

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'SPECIFIED OBJECT POINT DOES NOT EXIST'

C        RAYCOD(1)=16

C        RAYCOD(2)=I

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'NO DEFORM FILE EXISTS FOR THIS SURFACE'

C        'OR IT HAS INSUFFICIENT DATA FOR THIS SURFACE'

C        RAYCOD(1)=17

C        RAYCOD(2)=I

C

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'RAY COORDINATE BEYOND DEFORMABLE SURFACE BOUNDS'

C        RAYCOD(1)=18

C        RAYCOD(2)=I

C

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'AIMED CHIEF RAY COULD NOT HIT DEFINED TARGET POINT'

C        RAYCOD(1)=19

C        RAYCOD(2)=I

C

C

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'TIR CONDITION NOT MET'

C        RAYCOD(1)=20

C        RAYCOD(2)=I

C

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'HOE-R SOURCE CONFIGURATION RAY FAILURE'

C        RAYCOD(1)=21

C        RAYCOD(2)=I

C

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'HOE-R REFERENCE CONFIGURATION RAY FAILURE'

C        RAYCOD(1)=22

C        RAYCOD(2)=I

C

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'REFERENCE RAY COULD NOT INTERSECT THE NSS SURFACE'

C        RAYCOD(1)=23

C        RAYCOD(2)=I

C

C

C        'RAY FAILURE OCCURRED AT SURFACE ',I

C        'RAY COULD NOT INTERSECT THE NSS SURFACE'

C        RAYCOD(1)=24

C        RAYCOD(2)=I

C